Air Conditioning Project

Team Members:

Alaa Ibrahim

Bassel Yasser

Sharpel Malek

Sherif Khadr

Contents

INTRODUCTION	1

High Level Design	2

01)	Layered Architecture	2

02)	Modules Description	3

03)	Drivers’ Documentation	4

Low Level Design	14

Figure 1: Project Layered Architecture	2

# INTRODUCTION

# High Level Design

## Layered Architecture

Figure 1: Project Layered Architecture

## Modules Description

MCAL Layer:

- DIO: For controlling GPIO pins

- External Interrupt: this module takes place in MCAL layer. it controls three external hardware interrupts on pins PD2, PD3, and PB2 which are referred to as INT0, INT1, and INT2 respectively. External interrupts can be level-triggered or edge-triggered.
We program this triggering. INT0 and INT1 can be level-triggered and edge-triggered whereas INT2 can be only edge-triggered.

- Timer: Provides an interface with timer 0 low-level capabilities.

- ADC:

HAL Layer:

- Button:

- Keypad:

- LCD: Use for display data

- HTimer: Provides high-level functions using the lower level timer 0 module capabilities.

- HExtInt: Provides high-level functions using the lower level External interrupt module capabilities.

Service Layer:

Application Layer:

## Drivers’ Documentation

MCAL Layer:

- DIO:

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8SETPinDir

* Description	: Set Pin Direction

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- enPortDir {INPUT , OUTPUT}

* Return		:   Sint8_t

*/

Sint8_t DIO_s8SETPinDir (enu_pin enPinCopy, enu_dir enPortDir)

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8SETPinVal

* Description	: Set Pin Value

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- enPortDir {HIGH , LOW}

* Return		:   Sint8_t

*/

Sint8_t DIO_s8SETPinVal (enu_pin enPinCopy, enu_val enPortVal)

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8GETPinVal

* Description	: Set Pin Value

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- pu8Val address of variable that u want to save value on it

* Return		:   Sint8_t

*/

Sint8_t DIO_s8GETPinVal (enu_pin enPinCopy, Uint8_t* pu8Val)

- External Interrupt:

// EXT_INT TYPEDEFS

typedef enum EN_EXTINT_ERROR {

EXTINT_OK=0,

EXTINT_NOT_OK

}EN_EXTINT_ERROR;

typedef enum EN_Sence_Control {

LOW_LEVEL=0,

FALLING_EDGE,

RISING_EDGE,

ANY_LOGICAL_CHANGE

}EN_Sence_Control;

typedef enum EN_EXINT_NUMBER{

EXTINT0=0,

EXTINT1,

EXTINT2,

}EN_EXINT_NUMBER;

typedef enum EN_GLOBAL_INT{

DISABLE=0,

ENABLE

}EN_GLOBAL_INT;

// EXT_INT prototypes

/*

Description : This function initializes the GLOBAL_INTERRUPT

ARGS        : takes the state ( ENABLE OR DISABLE )

return      : return EXTINT_OK if the PIN initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR SET_GLOBAL_INTERRUPT(EN_GLOBAL_INT state);

/*

Description : This function initializes the external interrupt number and it's detecting type

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and sense control.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR EXTINT_init(EN_EXINT_NUMBER INTx ,EN_Sence_Control INTxSense);

/*

Description : This function takes the external interrupt number and initialize call back function.

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and pointer to the function we want to execute.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR EXTINT_CallBack(EN_EXINT_NUMBER INTx,void(*ptrfunc)(void));

- Timer:

/**

* \brief Initialize the timer with given mode

* \param u8_a_Mode

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_voidInit(en_TIMMode_t u8_a_Mode);

/**

* \brief Start the timer clock after prescaling it with given value

* \param u8_a_prescaler

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_Start(en_TIM_CLK_SELECT_t u8_a_prescaler);

/**

* \brief Function to stop timer 0

* \return void

*/

void TIM0_Stop();

/**

* \brief Set the timer to start from a certain value

* \param u8_a_FlagValue The value to start the timer from

* \return void

*/

void TIM0_SetValue(Uchar8_t u8_a_startValue);

/**

* \brief Function to get the value of the overflow flag of timer 0

* \param u8_a_FlagValue reference to a variable to store flag value *

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetOVF(Uchar8_t* u8_a_FlagValue);

/**

* \brief Function to clear timer 0 overflow flag

* \return void

*/

void TIM0_ClearOVF(void);

/**

* \brief Function to get the timer state (running/stopped)

* \param u8_a_State reference to a variable to store timer state

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetState(en_TIMState_t* u8_a_State);

/**

* \brief Function to set a function to call when the timer0

*	      Overflow Interrupt is triggered

* \param pv_a_CallbackFn reference to the function to call

* \return en_TIMErrorState_t

*/

HAL Layer:

Button:

/**

* \brief : This Function Use To Initialize Button Pin And Port

*

* \param ST_PUSH_BTN_t  *btn

*

* \return Std_ReturnType

*/

Std_ReturnType PUSH_BTN_intialize(const ST_PUSH_BTN_t *btn)

/**

* \brief : This Function Will Use To Read Button Statues If It Is Pressed Or Releassed

*

* \param ST_PUSH_BTN_t  *btn

* \param EN_PUSH_BTN_state_t  *btn_state

*

* \return Std_ReturnType

*/

Std_ReturnType PUSH_BTN_read_state(const ST_PUSH_BTN_t *btn , EN_PUSH_BTN_state_t *btn_state)

- Keypad:

- HTimer:

/**

* \brief Generate Synchronous delay (busy waiting)*

* \param Copy_delayTime Desired delay

* \param Copy_timeUnit Time units (Seconds, mSeconds, uSeconds)

*

* \return en_HTIMErrorState_t

*/

en_HTIMErrorState_t TIM0_SyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit);

/**

* \brief Generates delay asynchronously

* \param u32_a_delay desired delay

* \param u8_a_timeUnit delay time units

* \param Copy_pvCallbackFn function to call when delay is complete

*

* \return en_TIMErrorState_t

*/

en_HTIMErrorState_t TIM0_AsyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit, void (*Copy_pvCallbackFn)(void));

/**

* \brief Function to end a delay asynchronously

* To Stop Async Delay: No Restrictions

* To Stop Sync Delay: should only be called in an ISR/Callback function

*

* \return void

*/

void TIM0_AsyncEndDelay();

- HExtInt:

/*

Description : This function initializes the external interrupt number and it's detecting type and initialize call back function.

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and sense control and and pointer to the function we want to execute when interrupt occurs.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR H_EXTINT_create(EN_EXINT_NUMBER INTx ,EN_Sence_Control INTxSense,void(*ptrfunc)(void));

- HLCD:

/*

* function		: HLCD_vidInit

* description 	: func to set LCD initialization

* input param 	: void

* return		: void

* */

void HLCD_vidInit(void)

/*

* function		: HLCD_vidWritecmd

* description 	: func to configure some commands on lcd

* input param 	:

* 					u8commandCopy --> take lcd cmd instructions from instruction table <https://components101.com/sites/default/files/component_datasheet/16x2%20LCD%20Datasheet.pdf>

* return		: void

* */

void HLCD_vidWritecmd(Uint8_t u8commandCopy)

/*

* function		: HLCD_vidWriteChar

* description 	: func to write char on lcd

* input param 	: u8CharCopy -> take ascii code of char   or   char address on CGROM

* return		: void

* */

void HLCD_vidWriteChar(Uint8_t u8CharCopy)

/*

* function		: HLCD_ClrDisplay

* description 	: func to clear anything on lcd

* input param 	: void

* return		: void

* */

void HLCD_ClrDisplay(void)

/*

* function		: HLCD_gotoXY

* description 	: func to determine position which char print at this position on lcd  ### NOTE : (2rows x 16coloms)

* input param 	:

* 				  row -> take row number 0 or 1

* 				  pos -> take colom number from 0 ~ 16

* return		: void

* */

void HLCD_gotoXY(Uint8_t row, Uint8_t pos)

/*

* function		: HLCD_WriteString

* description 	: func to write string on lcd

* input param 	: str --> which take string as argument

* return		: void

* */

void HLCD_WriteString(Uint8_t* str)

/*

* function		: HLCD_WriteInt

* description 	: func to write integer number on lcd

* input param 	: number --> which take number as argument

* return		: void

* */

void HLCD_WriteInt(Uint32_t number)

/*

* function		: HLCD_vidCreatCustomChar

* description 	: func to store new patterm on CGRAM

* input param 	:

* 				  pu8custom  -> take pointer to array which having LCD Custom Character Generated data ### take only 8 characters

* 				  u8Location -> determine location on CGRAM [0 ~ 8]

* return		: void

* */

void HLCD_vidCreatCustomChar(Uint8_t* pu8custom, Uint8_t u8Location)

Application Layer:

# Low Level Design

MCAL Layer:

- DIO:

Sint8_t DIO_s8SETPinDir (enu_pin enPinCopy, enu_dir enPortDir)

Sint8_t DIO_s8SETPinVal (enu_pin enPinCopy, enu_val enPortVal)

Sint8_t DIO_s8GETPinVal (enu_pin enPinCopy, Uint8_t* pu8Val)

- External Interrupt:

- Timer:

TIM0_Init

TIM0_Start

TIM0_Stop

HAL Layer:

- Button:

- Timer0

HTIM_SyncDelay

- LCD:

void HLCD_vidInit(void)

void HLCD_vidWritecmd(Uint8_t u8commandCopy)

void HLCD_vidWriteChar(Uint8_t u8CharCopy)

void HLCD_ClrDisplay(void)

void HLCD_gotoXY(Uint8_t row, Uint8_t pos)

void HLCD_WriteString(Uint8_t* str)

void HLCD_WriteInt(Uint32_t number)

void HLCD_vidCreatCustomChar(Uint8_t* pu8custom, Uint8_t u8Location)

Application Layer:

- APP_Init