Air Conditioning Project

Team Members:

Alaa Ibrahim

Bassel Yasser

Sharpel Malek

Sherif Khadr

Contents

INTRODUCTION	1

High Level Design	2

01)	Layered Architecture	2

02)	Modules Description	3

03)	Drivers’ Documentation	4

Low Level Design	14

Figure 1: Project Layered Architecture	2

# INTRODUCTION

# High Level Design

## Layered Architecture

Figure 1: Project Layered Architecture

## Modules Description

MCAL Layer:

- DIO: For controlling GPIO pins

- External Interrupt: this module takes place in MCAL layer. it controls three external hardware interrupts on pins PD2, PD3, and PB2 which are referred to as INT0, INT1, and INT2 respectively. External interrupts can be level-triggered or edge-triggered.
We program this triggering. INT0 and INT1 can be level-triggered and edge-triggered whereas INT2 can be only edge-triggered.

- Timer: Provides an interface with timer 0 low-level capabilities.

- ADC:

HAL Layer:

- Button:

- Keypad: Deal with a set of buttons arranged in a block. The 3 x 3 matrix keypad usually is used as input in a project

- LCD: Use for display data

- HTimer: Provides high-level functions using the lower level timer 0 module capabilities.

- HExtInt: Provides high-level functions using the lower level External interrupt module capabilities.

Service Layer:

Application Layer:

## Drivers’ Documentation

MCAL Layer:

- DIO:

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8SETPinDir

* Description	: Set Pin Direction

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- enPortDir {INPUT , OUTPUT}

* Return		:   Sint8_t

*/

Sint8_t DIO_s8SETPinDir (enu_pin enPinCopy, enu_dir enPortDir)

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8SETPinVal

* Description	: Set Pin Value

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- enPortDir {HIGH , LOW}

* Return		:   Sint8_t

*/

Sint8_t DIO_s8SETPinVal (enu_pin enPinCopy, enu_val enPortVal)

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8GETPinVal

* Description	: Set Pin Value

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- pu8Val address of variable that u want to save value on it

* Return		:   Sint8_t

*/

Sint8_t DIO_s8GETPinVal (enu_pin enPinCopy, Uint8_t* pu8Val)

- Timer:

/**

* \brief Initialize the timer with given mode

* \param u8_a_Mode

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_voidInit(en_TIMMode_t u8_a_Mode);

/**

* \brief Start the timer clock after prescaling it with given value

* \param u8_a_prescaler

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_Start(en_TIM_CLK_SELECT_t u8_a_prescaler);

/**

* \brief Function to stop timer 0

* \return void

*/

void TIM0_Stop();

/**

* \brief Set the timer to start from a certain value

* \param u8_a_FlagValue The value to start the timer from

* \return void

*/

void TIM0_SetValue(Uchar8_t u8_a_startValue);

/**

* \brief Function to get the value of the overflow flag of timer 0

* \param u8_a_FlagValue reference to a variable to store flag value *

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetOVF(Uchar8_t* u8_a_FlagValue);

/**

* \brief Function to clear timer 0 overflow flag

* \return void

*/

void TIM0_ClearOVF(void);

/**

* \brief Function to get the timer state (running/stopped)

* \param u8_a_State reference to a variable to store timer state

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetState(en_TIMState_t* u8_a_State);

/**

* \brief Function to set a function to call when the timer0

*	      Overflow Interrupt is triggered

* \param pv_a_CallbackFn reference to the function to call

* \return en_TIMErrorState_t

*/

- ADC

/* Struct Contain all adc information to config it */

typedef struct

{

void(*interruptHandler)(void);

EN_ADC_REFERENCE_SELECTION_BITS_t referenceSource;

EN_ADC_ADJUST_RESULT_t resultAdjust;

EN_ADC_PRESCALER_SELECTION_t prescalerDivision;

EN_ADC_EVENT_TRIGGER_SOUREC_t triggerSource;

}ST_ADC_CFG_t;

/**

* \brief  : This Function Use To Init The Adc It Set Bits For Prescaler , Refrence Source , event trigger resource and Adjust Resualt

*

* \param  : const ST_ADC_CFG_t *_adc

*

* \return : Std_ReturnType

*/

Std_ReturnType ADC_Init(const ST_ADC_CFG_t *_adc)

/**

* \brief  : This Function Use To Disable The ADC

*

* \param  : const ST_ADC_CFG_t *_adc

*

* \return : Std_ReturnType

*/

Std_ReturnType ADC_Deinit(const ST_ADC_CFG_t *_adc)

/**

* \brief  : This Function Is Used To Select ADC Channel

*

* \param  : const ST_ADC_CFG_t *_adc

* \param  : EN_ADC_CHANNEL_SELECTION_t _channel

*

* \return : Std_ReturnType

*/

Std_ReturnType ADC_SetChannel(const ST_ADC_CFG_t *_adc , EN_ADC_CHANNEL_SELECTION_t _channel)

/**

* \brief  : This Function Use To Start Conversion

*

* \param  : const ST_ADC_CFG_t *_adc

*

* \return : Std_ReturnType

*/

Std_ReturnType ADC_StartConversion(const ST_ADC_CFG_t *_adc)

/**

* \brief  : This Function Use To Polling On The ADC Flag To Return The Conversion Resualt

*

* \param  : const ST_ADC_CFG_t *_adc

* \param  : Uint16_t *_ConversionResult

*

* \return : Std_ReturnType

*/

Std_ReturnType ADC_GetConversionResult(const ST_ADC_CFG_t *_adc , Uint16_t *_ConversionResult)

/**

* \brief : This Function Use To Make All Operation Of The Adc

*

* \param : const ST_ADC_CFG_t *_adc

* \param : EN_ADC_CHANNEL_SELECTION_t _channel

* \param : Uint16_t *_ConversionResult

*

* \return Std_ReturnType

*/

Std_ReturnType ADC_Conversion(const ST_ADC_CFG_t *_adc , Uint16_t *_ConversionResult , EN_ADC_CHANNEL_SELECTION_t _channel)

HAL Layer:

- Keypad:

// Macros

#define R1     DIO_PINC_2

#define R2     DIO_PINC_3

#define R3     DIO_PINC_4

#define C1     DIO_PINC_5

#define C2     DIO_PINC_6

#define C3     DIO_PINC_7

// user defined datatypes

typedef enum EN_KEYPAD_BTNS

{

KEY_INCREAMENT=0,

KEY_DECREAMENT,

KEY_SET,

KEY_ADJUST,

KEY_RESET,

KEY_6,

KEY_7,

KEY_8,

KEY_9,

KEY_NOTHING

}EN_KEYPAD_BTNS;

// functions prototypes

/*************************************************************************************************************

*

Name : KEYPAD_init()

Description : This Function Initializes keypad pins (Rows are outputs & Columns are inputs).

ARGS : void

return : void

************************************************************************************************************/

void KEYPAD_init(void);

/*************************************************************************************************************

*

Name : KEYPAD_GetButton

Description : This Function loops over other three functions (Checks (R1,R2,R3)).

ARGS : void

return : the pressed key or Nothing pressed

************************************************************************************************************/

EN_KEYPAD_BTNS KEYPAD_GetButton(void);

/*************************************************************************************************************

*

Name : KEYPAD_checkR1 ,  KEYPAD_checkR2, KEYPAD_checkR3

Description : functions are checking the entire row if it pressed or not.

ARGS : void

return : the pressed key or Nothing pressed

************************************************************************************************************/

EN_KEYPAD_BTNS KEYPAD_checkR1(void);

EN_KEYPAD_BTNS KEYPAD_checkR2(void);

EN_KEYPAD_BTNS KEYPAD_checkR3(void);

- HTimer:

/**

* \brief Generate Synchronous delay (busy waiting)*

* \param Copy_delayTime Desired delay

* \param Copy_timeUnit Time units (Seconds, mSeconds, uSeconds)

*

* \return en_HTIMErrorState_t

*/

en_HTIMErrorState_t TIM0_SyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit);

/**

* \brief Generates delay asynchronously

* \param u32_a_delay desired delay

* \param u8_a_timeUnit delay time units

* \param Copy_pvCallbackFn function to call when delay is complete

*

* \return en_TIMErrorState_t

*/

en_HTIMErrorState_t TIM0_AsyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit, void (*Copy_pvCallbackFn)(void));

/**

* \brief Function to end a delay asynchronously

* To Stop Async Delay: No Restrictions

* To Stop Sync Delay: should only be called in an ISR/Callback function

*

* \return void

*/

void TIM0_AsyncEndDelay();

- HLCD:

/*

* function		: HLCD_vidInit

* description 	: func to set LCD initialization

* input param 	: void

* return		: void

* */

void HLCD_vidInit(void)

/*

* function		: HLCD_vidWritecmd

* description 	: func to configure some commands on lcd

* input param 	:

* 					u8commandCopy --> take lcd cmd instructions from instruction table <https://components101.com/sites/default/files/component_datasheet/16x2%20LCD%20Datasheet.pdf>

* return		: void

* */

void HLCD_vidWritecmd(Uint8_t u8commandCopy)

/*

* function		: HLCD_vidWriteChar

* description 	: func to write char on lcd

* input param 	: u8CharCopy -> take ascii code of char   or   char address on CGROM

* return		: void

* */

void HLCD_vidWriteChar(Uint8_t u8CharCopy)

/*

* function		: HLCD_ClrDisplay

* description 	: func to clear anything on lcd

* input param 	: void

* return		: void

* */

void HLCD_ClrDisplay(void)

/*

* function		: HLCD_gotoXY

* description 	: func to determine position which char print at this position on lcd  ### NOTE : (2rows x 16coloms)

* input param 	:

* 				  row -> take row number 0 or 1

* 				  pos -> take colom number from 0 ~ 16

* return		: void

* */

void HLCD_gotoXY(Uint8_t row, Uint8_t pos)

/*

* function		: HLCD_WriteString

* description 	: func to write string on lcd

* input param 	: str --> which take string as argument

* return		: void

* */

void HLCD_WriteString(Uint8_t* str)

/*

* function		: HLCD_WriteInt

* description 	: func to write integer number on lcd

* input param 	: number --> which take number as argument

* return		: void

* */

void HLCD_WriteInt(Uint32_t number)

/*

* function		: HLCD_vidCreatCustomChar

* description 	: func to store new patterm on CGRAM

* input param 	:

* 				  pu8custom  -> take pointer to array which having LCD Custom Character Generated data ### take only 8 characters

* 				  u8Location -> determine location on CGRAM [0 ~ 8]

* return		: void

* */

void HLCD_vidCreatCustomChar(Uint8_t* pu8custom, Uint8_t u8Location)

Application Layer:

# Low Level Design

MCAL Layer:

- DIO:

Sint8_t DIO_s8SETPinDir (enu_pin enPinCopy, enu_dir enPortDir)

Sint8_t DIO_s8SETPinVal (enu_pin enPinCopy, enu_val enPortVal)

Sint8_t DIO_s8GETPinVal (enu_pin enPinCopy, Uint8_t* pu8Val)

- Timer:

TIM0_Init

TIM0_Start

TIM0_Stop

- ADC

- ADC_Init

- ADC_Deinit

- ADC_SetChannel

- ADC_StartConversion

- ADC_GetConversionResult

- ADC_Conversion

HAL Layer:

- Timer0

HTIM_SyncDelay

- LCD:

void HLCD_vidInit(void)

void HLCD_vidWritecmd(Uint8_t u8commandCopy)

void HLCD_vidWriteChar(Uint8_t u8CharCopy)

void HLCD_ClrDisplay(void)

void HLCD_gotoXY(Uint8_t row, Uint8_t pos)

void HLCD_WriteString(Uint8_t* str)

void HLCD_WriteInt(Uint32_t number)

void HLCD_vidCreatCustomChar(Uint8_t* pu8custom, Uint8_t u8Location)

- Keypad :

Application Layer:

- APP_Init